# Current pRF bar mapping design

Updated protocol • 4 October 2026

This protocol maps pRF positions around the region used for the illusion experiment. The current run lasts 348 seconds (5 minutes 48 seconds), or 174 volumes at TR = 2 seconds. The bar is 2° wide and moves at approximately 0.56° per second. The stimulus is restricted to a fixation-centered circle whose radius reaches the right screen edge; the physical screen clips the circle further.

# Display and viewing geometry

| Parameter | Current setting |
| --- | --- |
| Display | 1024 × 768 pixels, 60 Hz, requested 32-bit mode |
| Projected image | 354 mm wide × 265 mm high |
| Viewing distance | 1137 mm = 127 mm eye to mirror + 1010 mm mirror to screen |
| Pixels per degree | 57.4086, using PsychoPy standard deg approximation |
| Visible angular field | Approximately 17.84° wide × 13.38° high |
| Fixation position | 4° from left screen edge, vertically centered |
| Fixation offset | Nominal −4.92° from center; rounded offset [−282, 0] pixels |
| Fixation screen coordinates | Approximately [230, 384] pixels |
| Coverage relative to fixation | Approximately x = −4° to +13.84° and y = −6.69° to +6.69° |
| Left stimulus boundary | 4° left of fixation; subpixel differences arise from rounding |
| Canvas | 1588 × 1588 display pixels; diameter 27.66°, radius 13.83° |

The aperture radius is the horizontal distance from the rounded fixation position to the right screen edge: 794 display pixels, or approximately 13.83°. Its diameter and canvas are 1588 pixels. It touches the right edge at fixation height and excludes the upper-right and lower-right corners. Displayed and saved fitting masks share this circular support and the 4° left-coverage limit.

# Calibration convention

Pixels per degree are computed as viewing distance × 0.017455 × horizontal pixels ÷ projected image width. Fixation offset is computed from screen width and the requested left-edge distance. Positive x is right; positive y is up in the entered degree coordinates, and y is reversed for display pixel offsets.

These values use the same width-based linear calibration on both axes. Projected height is recorded but is not used to calculate a separate vertical scale. The angular values are not exact perspective-corrected angles; verify that the main illusion stimulus uses the same calibration convention.

# Bar stimulus and fixation task

| Parameter | Current setting |
| --- | --- |
| Bar width | Fixed 2°; approximately 114.82 display pixels |
| Width relative to visible field | Approximately 11.2% of the 17.84° horizontal extent; no longer exactly 1/8 |
| Common movement speed | Approximately 0.56° per second across all directions |
| Point exposure per sweep | Approximately 3.60 seconds when fully crossed |
| Movement per 2-second TR | Approximately 1.11°, leaving overlapping stimulus coverage |
| Untrimmed trajectory | 838 movie frames or 55.867 seconds; invisible ends are trimmed |
| Movie update rate | 15 Hz; four refreshes per movie frame on the 60-Hz display |
| Run frames | 5220 planned movie frames |
| Sweep directions | Eight, presented in a fixed order |
| Visual texture | Original colorful object textures, randomly selected during playback |
| Background | Gray, uint8 value 127 |
| Movie flips | [0, 0], with no horizontal or vertical flip |

The approximately 3.60-second exposure is a visual coverage duration, not a BOLD duration. It is computed from bar width divided by movement speed. Coverage can be shorter at clipped boundaries; the response of a voxel also depends on its pRF size and the hemodynamic response function.

# Fixation and overlay

The fixation dot is 4 display pixels, with red, black and white colors and alpha 0.5. Its color changes around every 3 seconds with a ±2-second range, quantized to movie frames. Sweep order is fixed; texture selection and fixation-change timing are randomized. The fixation task continues during rests and padding.

The overlay is generated from calibrated ring radii of 1°, 2°, 5° and 10°, centered on fixation. Eight radial spokes are retained at 45° increments. Lines are white with antialiasing, approximately one display pixel wide and maximum alpha 45/255. The center is cleared within a 4-pixel radius for the fixation dot. The overlay is clipped by the screen.

# Reference comparison

Benson et al. (2018), the HCP 7T retinotopy study, used a circular field of 16° diameter, a 2° bar, 28-second traversals followed by 4-second blanks, and reported 3.11 seconds of exposure per point. The implied speed is approximately 0.64° per second. Our current bar has the same absolute width and slightly longer point exposure. HCP used a 1-second TR and two bar runs within a six-run mapping session; its results do not validate a single run or our exact off-center coverage.

Reference: https://doi.org/10.1167/18.13.23

# Complete block schedule

Each sweep block includes its listed blank padding. Padding follows the bar presentation and is additional to the scheduled rest. Every block ends on a whole TR. Timing uses integer movie frames; decimal seconds below are rounded.

| Block | Time s | Bar s | Padding s | Total s | TRs |
| --- | --- | --- | --- | --- | --- |
| Initial rest | 0–12 | — | — | 12 | 6 |
| Left → right | 12–48 | 35.60 | 0.40 | 36 | 18 |
| Rest | 48–52 | — | — | 4 | 2 |
| Down → up | 52–80 | 28.00 | 0.00 | 28 | 14 |
| Rest | 80–84 | — | — | 4 | 2 |
| Right → left | 84–120 | 35.60 | 0.40 | 36 | 18 |
| Rest | 120–124 | — | — | 4 | 2 |
| Up → down | 124–152 | 28.00 | 0.00 | 28 | 14 |
| Rest | 152–156 | — | — | 4 | 2 |
| Lower-left → upper-right | 156–198 | 41.33 | 0.67 | 42 | 21 |
| Rest | 198–202 | — | — | 4 | 2 |
| Lower-right → upper-left | 202–244 | 41.33 | 0.67 | 42 | 21 |
| Rest | 244–248 | — | — | 4 | 2 |
| Upper-right → lower-left | 248–290 | 41.33 | 0.67 | 42 | 21 |
| Rest | 290–294 | — | — | 4 | 2 |
| Upper-left → lower-right | 294–336 | 41.33 | 0.67 | 42 | 21 |
| Final rest | 336–348 | — | — | 12 | 6 |

Totals: 292.53 seconds of bar presentation + 3.47 seconds of padding + 52 seconds of scheduled rest = 348 seconds. Maximum padding is 10 movie frames, or 0.667 seconds. Initial and final rests are each 12 seconds; there are seven intermediate rests of 4 seconds.

# Timing implementation

ptviewmovie_prf93 schedules movie frames against absolute deadlines anchored to the first movie onset, using a fixed 4/60-second frame interval. Late frames do not shift all subsequent targets. It holds the final frame through the planned run end and preserves early-abort behavior. It records target times, actual flip errors, missed-flip values, late-frame count and an overall timing flag.

Keep synchronization tests enabled. A prior 448-second version was reported to finish at 448.010 seconds with zero late frames after this correction; that does not constitute a timing test of the latest 356-second design. Confirm the latest playback duration and diagnostics on the actual acquisition display.

# Saved files and fitting workflow

Run files are saved inside C:\Users\TseLab\Desktop\pRF_stim\subjN\, where N is entered at the subject prompt. Filenames contain a timestamp, subject number, run number and experiment number 93. The stimulus starts on trigger key 5. PNG capture is currently disabled to avoid screen-readback and file-writing overhead.

| Output | Contents |
| --- | --- |
| Main run MAT file | Planned frame order, actual frame/key timestamps, run parameters and playbackTiming diagnostics |
| Companion stimulus MAT file | Native 768 × 768 mask bank, mask indices, frame order, screen/canvas geometry, offsets and metadata |
| nominalStimulus | Approximately 278 × 278 × 174; coverage averaged over each planned 2-second TR |
| gazeClippedMaskBank | Masks resized with bilinear interpolation and screen/left-boundary clipping applied |
| xDVA and yDVA | Fixation-centered coordinates and degrees-per-pixel calibration |
| visibleAnalysisMask | Visible domain for inspection or optional fitting constraints |
| apertureInfo | Actual analysis pixels per degree, display geometry, TR and timing basis |

AnalyzePRF can use these arbitrary aperture masks; the partially visible circle is represented explicitly, including its excluded corners. The analysis origin is fixation at (0°, 0°). Do not apply the screen-center fixation offset again. Outside-view locations, scheduled rests and padding have zero stimulus coverage. Each TR averages 30 planned movie frames; it is not a single beginning, midpoint or end snapshot. The static fixation overlay is not included in the bar-aperture model.

For fitting, use double(nominalStimulus) and preprocessed BOLD with matching time points and run boundaries. Align the trigger, dummy-volume policy and retained BOLD samples explicitly. Convert fitted eccentricity and pRF size from analysis pixels to degrees using apertureInfo.analysisPixelsPerDegree. Use polar angle to obtain x and y when needed.

# Scientific scope and interpretation

The priority is reliable pRF positions around the illusion location, rather than precise recovery at distant screen boundaries. Check that this region is crossed by every sweep and assess x/y reliability across repeated runs. pRFs extending beyond the screen can have uncertain centers and sizes even when the visible masks are correct. Good fit quality alone does not establish parameter accuracy.

Zeroed regions contribute no stimulus drive, but analyzePRF fits a complete Gaussian pRF. No visible-only center constraint has been added. The exported movie follows planned timing; actual-time reconstruction remains to be implemented if recorded timing departs materially from the schedule.

# Automatic and manual settings

The circular aperture radius follows the distance from fixation to the right screen edge automatically. Screen clipping and the left-coverage limit are applied to the masks. The bar width is explicitly fixed at 2°. The fitting grid targets about 0.1° per pixel, currently 278 × 278. Sweep trimming and whole-TR padding are recalculated from masks. The current 838-frame full trajectory was selected for this geometry; geometry changes require a new speed/padding search. Earlier saved stimulus files remain unchanged.

Custom files: runretinotopy.m, prepare_prf93.m, showmulticlass_prf93.m, ptviewmovie_prf93.m, save_prf_apertures.m and ptoff.m, in the knkutils-master\pt folder. The analyzePRF fitting algorithm itself has not been modified.